- Principle

Red cells with a positive DAT cannot be tested accurately with blood typing reagents that require an indirect antiglobulin technique.  Chloroquine diphosphate dissociates IgG from the red cell membrane with little or no damage to its integrity if used under controlled conditions.

- Scope and Related Policies

All reagents prepared in-house that contain a controlled substance must be labeled with a workplace label as per WHMIS legislation.9.1

- Specimen – N/A

- Material

Equipment:		Balance

Supplies:		Weigh boat

Glass reagent bottle

Glass beaker or flask

Serological pipettes

pH strips

Reagents:		Saline

1N NaOH

Chloroquine Disphosphate salt

- Quality Control

Serological pipettes should be maintained as per manufacturers recommendations including adequate volume delivery, reduction of carryover and absence of contamination. 9.2

- Procedure

- Dissolve 20 g of chloroquine diphosphate in 100 mL of saline.

- Adjust pH to 5.1 with 1 N NaOH.

- Store in labeled glass reagent bottle at 2-8°C for six months.

- Reporting  – N/A

- Procedural Notes – N/A

- References

  - WHMIS www.whmis.ca.

  - CSTM Standards for Hospital Transfusion Services, ver 3 February   2011:3.4.5.1.

  - Fung MK Ed. Technical Manual 18th Edition. AABB Press Bethesda MD; 2014 Method (2-20).

- Revision History

| Revision Date | Summary of Revision |
| --- | --- |
| December 1, 2014 | - Revised name of manual - Added sections 2.0 & 5.0 - Changed 2-6°C to 2-8°C in section 6.3 - Updated list of references |